P202 – See Description Below

Pin8 -------------------------Pin 1

J201 – A, B, and C

Stepper Lines

J202 – A, B, and

C Limit Switches

P204 – X Limit

Switch

P203:Aux GRBL

connectors:Reset, Feed

Hold, Cycle Start, Safety Door

J203 – Y and

Z Limit Switches

P201  +12V Supply

P202 Connection Description (Right to Left):

Pin 1:	~ENABLE

Pin 2:	~Z-Dir

Pin 3:	~X-Dir

Pin 4:	~Y-Dir

Pin 5:	~Z-Step

Pin 6:	~X-Step

Pin 7:	~Y-Step

Pin 8:	+5V

CAUTION – A4988 Stepper Controllers must be oriented so the VMOT pin is close to the electrolytic capacitor!!!!!

Turn to the next page for More Wonderfulness . . .

SW101, SW102, and SW103 set the micro-stepping for the A4988 stepper controllers:

0 = Full Step, 1 = Half Step, 2 = Quarter Step, 3 = Eighth Step, 7 = Sixteenth Step

Green LED indicates +5V working, Red LED indicates +12V working

Pin-outs for the J201, J202, and J203 Connectors: